Typical PYNQ Development Steps

NOTE: PYNQ library only simplifies HOST (CPU) programming (Python programming instead of low-level C programming) of an SoC / CPU-FPGA system, and it doesnl change the FPGA hardware development flow itself. You will need to create FPGA design as before (with Vivado). (<ad time starts> OR, use PyLog to create your FPGA design! <ad time ends>)

- Complete FPGA synthesis and generate bitstream, get the following two files:

- Bitstream (*.bit), you can search for it with command: find . -name “*.bit”. Its typical location is: ,/{project_name}/{project_name}.runs/impl_1/design_1_wrapper.bit

- Hardware Handoff File (*.hwh): search: find . -name M*.hwhM. You may find multiple *.hwh file, the right one is: 7(project_name}/(project_name).srcs/sources_1/bd/design_1/hw_handoff/design _1.hwh

- Copy the bitstream file and hardware handoff file to ZedBoard. (You may also want to rename these files so that they have a meaningful name)

- Create a Python script that loads the bitstream, allocates memory and interacts with FPGA.

- Run the Python script with “sudo python3.6 {your_python_script).pyH. Note: pynq module requires sudo to import.

You can find example Python code here: httDS：//drive.QOoale.com/drive/folders/1fmOanJ8SBilMNbKRfhSZu8IKwhe1k 9n

- zedboard_test.py: This is the example host code

- pylog_add.bit: bitstream

- pylog_add.hwh: hardware handoff file

- (not needed, only for reference) pylog_add.cpp: C code used for high-level synthesis (generated by PyLog)

- (not needed, only for reference) vecadd.py: PyLog source file that generates C code

REFERENCES

- PYNQ Documents: httDS：//Dvna.readthedocs.io/en/v2.5.1/

- PYNQ-Z1 Board Setup Guide:

https://pynq.readthedocsjo/en/v2.5.1/qettinq started/pynq z1 setup.html